Electronics lab 4

Calculate value for R5 when we want to have the current that goes through the Stanley MPY3362X to be 10 mA when it is being driven from Raspberry PI Pico’s digital output pin in High (1) state. Digital output pin is in the default 4mA settings.

LED’s forward voltage at 10mA 2V

Rasperry PI Pico’S output voltage at 10mA output current with default 4mA setting: 2.85V

Resistance value needed for R5:   85W

Obtain suitable Resistor from the teacher and build the circuit. Measure voltages VR5: 0,90V and VLED: 1,98 V. How much was the current I1: 9,6 mA

Investigate Your Digital Multi Meters user manual  in  electronics/datasheets directory in OMA and also look at what information is available in the biltema’s  website. Find out manufacturer specified measurement uncertainty for following measurement ranges

| Range | Resolution | measurement uncertainty (accuracy or tarkkuus in user manual) |
| --- | --- | --- |
| DC 200 mV | 100 uV | +- 0,5% +- 2 num |
| DC 2000 mV | 1 mV | +- 0,5% +- 2 num |
| DC 20 V | 10 mV | +- 0,5% +- 2 num |
| DC 200 V DC 300 V | 100 mV 1 V | +- 0,5% +- 2 num +- 0,5% +- 2 num |
| AC 200V | 100 mV | +- 1,2% +- 10 num |
| AC 300V | 1 V | +- 1,2% +- 10 num |
| Current DC 2000 uA | 1 uA | +- 1% +- 2 num |
| Current DC 20 mA | 10 uA | +- 1% +- 2 num |
| Current DC 200 mA | 100 uA | +- 1,2% +- 2 num |
| Current DC 10 A | 10 mA | +- 2% +- 2 num |
| Resistance 200 ohm | 100 mOhm | +- 0,8% +- 2 num |
| Resistance 2000 ohm | 1 Ohm | +- 0,8% +- 2 num |
| Resistance 20 kohm | 10 Ohm | +- 0,8% +- 2 num |
| Resistance 200 kohm | 100 Ohm | +- 0,8% +- 2 num |
| Resistance 2000 kohm | 1 kOhm | +- 1% +- 2 num |

Calculate what the meter can show if we are measuring a very precise 1 V voltage reference that has a zero output resistance with the meter on following different DC ranges:

| Range | minimum possible reading when measuring 1v reference (V) | maximum possible reading when measuring 1v reference (V) |
| --- | --- | --- |
| DC 200 mV | 0,9948 | 1,0052 |
| DC 2000 mV | 1-1*0.005-2*0.001 = 0,993 | 1,007 |
| DC 20 V | 0,975 | 1,025 |
| DC 200 V DC 300 V | 0,795 -1,005 | 1,205 3,005 |

What is the input resistance of the DMM ?

| Range | Specified input resistance | measured input resistance |
| --- | --- | --- |
| DC voltage | Not shown in the manual | 0,97 MOhm |

You can measure the input resistance using following circuit.  Measure U, Vr7 => calculate Vr6

Calculate I and Ir7   => calculate Irdmm     => calcite internal resistance Rdmm

U=3,33

Vr7 = 1,17

| U | Vr7 | Vr6 | I |
| --- | --- | --- | --- |
| 3,33 V | 1,17 V | 2,16 V | 2,62 uA |
| Ir7 | IRdmm | Rdmm |  |
| 1,42 uA | 1,2 uA | 0,97 MOhm |  |

What is the input resistance of your DMM when measuring DC current? (Look at “Mittauksen jännitehäviö” and calculate from that value from the user manaual)

| Range | Specified input resistance | Measured input resistance |
| --- | --- | --- |
| Current DC 2000 uA | 100 | 104,0 |
| Current DC 20 mA | 10 | 14,0 |
| Current DC 200 mA | 1 | 5,6 |
| Current DC 10 A | 0,01 | OL |